Master’s Thesis

Point- vs. diffuse-source pollution: 
uncertainty-aware attribution analysis in Mediterranean agricultural river basins with sparse water quality data.

Author: Konstantin Pascal Walter

Supervisor: Cesar Rubén Fernandez De Villaran San Juan

Acknowledgements:

As most scientific work, this thesis rests on collective effort for data, models, and discussion.

Grateful acknowledgment is made to Rubén Fernández de Villarán   for supervision and substantial model setup help and sharing, to Francisco José Rueda Valdivia   for sustained day-to-day guidance, and to Laura Galván  for key contributions to the calibrated hydrology–sediment configuration forming the basis of this work.

Thanks also go to colleagues, friends and family for constructive discussions, and to the data custodians and public agencies cited in Chapter 3 for making the underlying datasets available. Any remaining errors or omissions are the sole responsibility of the author.

This work was Supported by grant TED2021-130744B-C22 and funded by MICIU/AEI/10.13039/501100011033 and by the European Union next generation EU/PRTR.

Abstract
In sensitive Mediterranean watersheds, credible water-quality assessment begins with data quality—coverage, comparability, and explicit uncertainty. Under rapidly changing hydro-climatic and anthropogenic conditions, an uncertainty-propagating model is needed to attribute point and diffuse nutrient sources. This thesis applies an uncertainty-aware SWAT workflow to the Cubillas sub-basin (Alto Genil), perturbing two data-rich inputs within defensible bounds: urban wastewater (point) and soil nutrient initialization (diffuse). Ensembles generated via a reproducible GIS–Python–SWAT pipeline are evaluated against observations using envelope widths (sensitivity), shifts relative to a BASE run (directional displacement), and regime diagnostics.

Results show pathway-specific responses: diffuse, soil-linked inputs yield event-driven phosphorus signals, while wastewater produces regime-spanning nitrogen responses; combined forcing is additive. In comparisons with observations, phosphorus envelopes—especially under combined forcing—capture a substantial share at low bias, whereas nitrogen agreement hinges on treatment of below-detection values. Making uncertainty explicit clarifies attribution, highlights remaining gaps, and provides concrete guidance for monitoring priorities and model refinement in data-limited, sensitive environments.

Keywords
nonpoint contamination, hydrological modelling, sparse monitoring, sensitivity, monte carlo, pipeline

Table of Contents:

# Introduction

The EU Water Framework Directive (Directive 2000/60/EC, 2000) is a cornerstone of European environmental law, aiming to protect and improve the quality of all water bodies. It requires member states to achieve at least “good ecological status” for rivers, lakes, groundwater, and coastal waters by 2027. In this context, understanding and predicting water quality—and how it responds to human-induced pressures and restoration measures—is critical and imperative. Yet this understanding faces multiple challenges. First, multiple variables are used to characterize water quality. For example, the US Geological Survey lists up to 24,898 variables across 17 categories (https://help.waterdata.usgs.gov/codes-and-parameters/parameters). Second, but not less importante,  the transport and fate of heat, dissolved, and suspended substances is controlled by a series of complex and interacting processes. These vary in space, time and under distinct environmental conditions. Moreover, available water-quality data are often sparse in space, inconsistent in-time, and limited. Most water-quality variables still require manual and labour-intensive measurements using ‘grab samples’ that are taken to the laboratory and analzed. There are sensors that can be used for routine, automated measurements, but only for a few water-quality variables (mainly temperature, dissolved oxygen, specific conductance, nitrite/nitrite or dissolved organic carbon, Yaroshenko et al. 2020). The number of sites where those routine-, high-frequency measurements are available, and, the length of the records is very limited. Given these multiple layers of complexity, understanding water quality can be a  challenging task (Zhi et al. 2023).

During the past few decades, our understanding of the physical, chemical, and microbiological processes governing biogeochemical cycles in inland waters has advanced substantially. However, our ability to predict water quality in inland water bodies in response to long- and short-term changes in the landscape, hydro-meteorological forcing, or management interventions remains limited. Consequently, significant challenges persist in managing, controlling, and improving river and reservoir water quality, and many problems continue to this day. In Spain, for example, nearly 90% of the more than 1,600 man-made reservoirs have been reported as eutrophic or at risk of eutrophication due to increased nutrient loads from urbanization, industrialization, and intensive agriculture (Rueda et al., 2024). Nutrient enrichment—primarily nitrogen and phosphorus— is evidenced by the increasing frequency of algal (including cyanobacterial) blooms, near-bottom hypoxia during summer stratification, and the subsequent mobilization of reduced substances, leading to serious consequences for drinking water, fisheries, and, and the overall integrity of freshwater ecosystems (Smith et al., 2018). Most efforts in eutrophication control have traditionally focused on reducing point-source nutrient loads sources (urban/industrial effluents) through the construction or upgrading of wastewater treatment facilities, as instructed in the European Council Directive 91/271/EEC (1991). But more recently, attention has shifted toward non-point (diffuse) sources (e.g. leaching, runoff and erosion from agricultural fields) through landscape changes and adoption of agricultural best management practices. However, the relative effectiveness of point- versus non-point source pollution controls remains uncertain. Evidence from long-term studies in the United States shows mixed and context-dependent trends in nutrient concentrations in inland waters, with variable success of non-point source mitigation measures and heterogeneous responses shaped by basin-specific characteristics, legacy nutrient stores, and implementation constraints (e.g., Tomczyk et al., 2023; Zhang et al., 2024). Specifically, the effectiveness of point versus non-point control strategies is likely to depend on the respective contributions of these sources to total nutrient loads prior to restoration. However, disentangling these contributions is inherently difficult, and becomes especially challenging in systems where monitoring networks are sparse and the length and resolution of the existing records limited relative to the spatial and temporal variability of hydrological and biogeochemical processes. This is the case for many river basins in southernmost Europe under semi-arid Mediterranean conditions, where sampling frequencies designed to comply with the EU Water Framework Directive are insufficient to capture the pronounced hydrological variability, characterized by extreme fluctuations across temporal scales, from synoptic (event-scale) and seasonal dynamics to strong interannual variability. In such contexts, a robust modelling-based source attribution analysis is required. Hydrological and water quality models are widely used as tools for water managers, planners, and researchers to understand and predict processes at the watershed scale (Tilahun Ashine, 2022). Their usefulness largely depends on how well they represent complex biophysical dynamics and how carefully their forcings and assumptions are handled. Yet even when leveraging sophisticated approaches, uncertainty inevitably permeates model predictions of nutrient loads due to the stochastic nature of forcing and state-variable data, as well as structural limitations inherent to the models themselves. Moreover, the representation of biogeochemical processes in landscapes requires parameterization, and parameter values are often poorly constrained or unknown. Uncertainty cannot be ignored and needs to be explicitly incorporated and quantitatively assessed in source-attribution analyses, as well as in the design, implementation, and evaluation of water quality restoration and control measures informed by modelling. The implications of model uncertainty must also be clearly communicated in project reports and forecasts (Rueda et al. 2024). Such practices can help sustain stakeholder engagement and mitigate perceptions of failure when restoration outcomes diverge from expectations. Even when data limitations and parameter uncertainty restrict predictive confidence at large scales, models can still be applied heuristically to explore system dynamics (e.g. MacLeod, (2016). These exploratory uses can yield important understanding and support decision-making. Elliott et al. (2020) for example propose a heuristic load-allocation method combining simple catchment models, load limits, and upstream–downstream relationships to infer source changes needed to meet targets.

Our goal is to quantify nutrient and organic carbon loads from point and non-point sources—including their relative magnitudes and temporal variability—within a river network draining a Mediterranean watershed with limited water quality data. Cubillas reservoir and its contributing watershed, in southern Spain, under Mediterranean climate (Fig. 1), is taken here as a case example. The available water quality data from government monitoring programs established to comply with the requirements of the European Water Framework Directive are very limited. Only one monitoring point exists upstream the reservoir, with 44 records of total phosphorus, for example, available in the 24-year period from 1998 to 2022. There are no records from 2000 to 2008, and the median frequency of sampling is quarterly. This frequency contrasts with the natural variability of inflows, with several major storm events ocurring only in fall and winter time. Total nutrient loading estimates, hence, are highly uncertain, the relative contributions of different sources being generally consistent at the order-of-magnitude scale.

The Soil Water Assessment Tool SWAT model (Arnold et al., 2012) will be used to simulate nutrient fluxes in the watershed over several decades within an uncertainty framework. The results will be used to identify the dominant pollution sources (point versus non-point), quantify key uncertainties in source attribution, and reconstruct the temporal history, trends, and variability of nutrient and organic carbon fluxes from synoptic to interannual scales. Given the limited availability of water quality data, model predictions are constrained and must rely on as much complementary information as possible, including qualitative and indirect evidence. In particular, sediment transport and erosion rate estimates are used to constrain biogeochemical model behaviour and reduce predictive uncertainty.

Based on the number of inhabitants in the watershed and assuming a per-capita wastewater production rate of approximately 300 L per person per day, with typical concentrations of 9±7 gPm-3 and 50±30 gNm-3 for untreated wastewater (Metcalf & Eddy 1995), point-source nutrient loads can be estimated at roughly 40 kgPd-1 and 200 kgNd-1.  These estimates are also of the same order of magnitude as those of non-point source loads. These, in turn, can be calculated from sediment loading estimates assuming that nutrients are carried in particulate form, i.e. adsorbed to sediment particles. This is particularly the case of P (e.g. Havlin 2004).  Sediment loads have been estimated from the volume of sediment accumulated in the reservoir and assuming that (1) sediments in the reservoir have a density of ca. 1590 kg/m3, and (2) sediment trap efficiency can be of up to 80% (see Ortega Murcia 2021). The volume of sediments are estimated in turn from series of bathymetric surveys in the reservoir at 0.12 hm3 year-1. Average sediment load estimates are at 3.7 Tn ha year-1 (or 2·105 T/year). Assuming average concentrations of P in soil of ca. 13 mgP/100g, calculated from the available soil pits in Cubillas catchment (Mapa de Suelos de la Provincia de Granada, Proyecto LUCDEME Ministerio de Medio Ambiente 2006; Ortega Murcia 2021), one gets 85 kgP d-1. This is almost twice as much the average load from point sources, but of the same order of magnitude. And both estimates are also of the same order of magnitude as others calculated from nutrient concentrations in the river, as reported by the River Basin Authorities (0.24 gP m-3, 2.4 gNm-3, Table X – Appendix B REFERENCIAS!!!), and publicly available inflow rates into the reservoir (1.56 m3s-1, SAIH webpage). Estimates of P and N loads in the river are roughly at 32 kgPd-1, and 324 kgNd-1. Note though that reported nutrient concentrations in rivers are estimated from samples collected only 2–3 times per year, mostly during dry periods between storm events, when river flows are below peak levels, when non-point source sediment and nutrient loads from agricultural fields are expected to be highest. In addition, flow rates in the Cubillas River at the sampling point are affected by diversions from and to neighbouring watersheds. In summary total nutrient load estimates from different sources vary considerably and are highly uncertain, the relative contributions of different sources are generally consistent at the order-of-magnitude scale.

This thesis is organized as follows. In section 2 (methodology) we will describe the Soil Water Assessment Tool SWAT (Arnold et al. 2021), with particular emphasis on the biogeochemical sub-models included to simulate the fate and transport of different forms of organic C and nutrients (N/P), and, the required input data (forcing/parameters). We will describe local data bases and results from previous studies used to guide model construction and to evaluate the goodness of model simulations. We will briefly describe procedures followed to evaluate forcing and model parameters, and their uncertainty (cross validation). In section 3 (results) we will first introduce already-calibrated results of the hydrologic routines, and projections (not sure if we will get there?). We will present simulations conducted with two different configurations of the water-sediment model, with similar performance (evaluated by comparing observed and simulated results) when comparing hydrological (flow rates) variables, but largely varying on their sediment erosion/transport outputs. We will analyze the consistency of soil information, by comparing datasets from different sources, and show the different maps of soil N/P content obtained through different interpolation approaches (varying in complexity from inverse distance squared to krigging). Montecarlo sampling methods will be used to evaluate the impact of input data uncertainty on model results, which will be compared against available measurement data sets of water quality.

This thesis aims to contribute to ongoing efforts of understanding water quality in Mediterranean rivers in response to increasing anthropogenic pressures related to land-use changes and climate variability. The major methodological contribution consists of a reproducible pipeline for the water quality routines of SWAT in which uncertainty in the input data is propagated to evaluate its impact on model outputs. Comparing SWAT predictions with observed data, one could establish whether the uncertainty ranges of the chosen sources are sufficient to reconcile model predictions with measurements, or whether unexplained knowledge gaps remain.